# Basic Computer Organization Project using Verilog and Proteus

## 1. Introduction

This project aims to design and implement a simple computer system using Verilog and simulate its electronic behavior using Proteus. The system performs basic operations such as the fetch-execute cycle, arithmetic and logical processing, and memory management through a central processing unit (CPU).

## 2. System Design

### Components of the System

- Memory:
256 locations × 16 bits each

- Registers:

  - IR (Instruction Register)

  - TR (Temporary Register)

  - DR (Data Register)

  - AC (Accumulator)

  - PC (Program Counter)

  - AR (Address Register)

  - time Count (Timing Counter)

- Additional Components:

  - I: Indirect Addressing Bit

  - E: Extend (Carry) Flip-Flop

  - S: Start/Stop Flip-Flop

## 3. Architecture Overview

The core of the system is the BasicComputer module, which contains a CPU that stores and executes instructions from memory based on the fetch-execute cycle.

### Execution Flow:

- Memory Initialization:
Instructions and data are preloaded into memory, and all registers are initialized.

- Fetch-Execute Cycle:
Instructions are fetched from memory and executed in a time-sequenced manner according to the contents of the registers.

## 4. Verilog Code Components

### Main Module: BasicComputer

The central processor is capable of executing:

- Register-Reference Instructions:
Such as CLA, CLE, INC, etc.

- Memory-Reference Instructions:
Such as AND, ADD, LDA, STA

- Addressing Modes:
Supports both direct and indirect addressing

### Key Implementation Features

- Memory Allocation:
The system includes 256 memory locations, each 16 bits wide

- Timing Logic:
A counter (time Count) is used to manage the sequential timing of operations

## 5. Testbench Simulation

A testbench is provided to simulate and verify the computer's operation:

- Generates clock pulses to drive the system

- Displays the values of key registers (IR, TR, DR, AC, PC, AR) at each clock cycle

- Exports simulation data to a VCD file for waveform visualization using tools like GTKWave

## 6. Project Features

- Fetch-Execute Cycle Simulation:
Accurately models the traditional instruction cycle in a CPU

- Memory and Data Handling:
Simulates loading and storing of data between memory and registers

- Instruction Execution:
Supports various types of instructions, including arithmetic and logic operations

- Proteus Simulation:
A schematic is created in Proteus to visually simulate and validate the hardware behavior

## 7. Conclusion

This project demonstrates the design and simulation of a basic computer at the hardware level using Verilog and Proteus. It provides practical insight into the structure and functioning of a CPU, including the fetch-execute cycle, memory operations, and instruction handling. It serves as a foundational step for understanding computer architecture and digital system design.